## Step 1: Importing Data into QGIS

- Start QGIS:

  - Open QGIS and create a new project.

- Add Shapefiles:

  - Method 1: Drag and drop each shapefile (e.g., tl_2024_36_tract.shp, NYC_Borough_Boundary.shp, nyc_parks.shp) directly into the QGIS Layers Panel.

  - Method 2:

    - Click on Layer > Add Layer > Add Vector Layer…

    - Browse to your file location, select the shapefile, and click Open.

  - Confirm that all three layers appear in your Layers Panel.

## Step 2: Check and Harmonize Coordinate Reference Systems (CRS)

- Inspect CRS:

  - Right-click on each layer (e.g., NY State Census Tracts) and select Properties.

  - Under the Information (or Source) tab, check the CRS details.

- Reproject if Necessary:

  - If the layers are in different CRS, you should reproject them to a common CRS (a good choice is the CRS used by the NYC Borough Boundaries).

  - To reproject a layer:

    - Go to Reproject layer (you can use the Toolbox to find this as well or go to Processing > Toolbox and then search for “Reproject Layer”

    - Put the CRS reference system that you want (usually to match another CRS system)

  - Add the newly reprojected layer to your project if it doesn’t automatically appear.

## Step 3: Clipping Census Tracts to NYC Borough Boundaries

- Open the Clip Tool:

  - Go to Processing > Toolbox to open the Processing Toolbox.

  - In the toolbox, navigate to Vector overlay and double-click on Clip.

- Set Clip Parameters:

  - Input Layer: Select the NY State Census Tracts layer.

  - Overlay Layer: Select the NYC Borough Boundary layer.

  - Output Layer: Choose to save the result as a temporary layer or specify an output file path (e.g., nyc_census_tracts.shp).

- Run the Clip:

  - Click Run.

  - A new layer (e.g., “Clip”) will be added to your Layers Panel showing only the census tracts that fall within the NYC borough boundaries.

- Visual Check:

  - Toggle the layers’ visibility to confirm that the census tracts are now limited to the NYC area.

Step 4: Fix Geometries (if you skip this, there will be an error in Step 5)

## The NYC Parks file has errors in the geometry that will stop us from merging it’s feature to the census tracts file. Thus we must fix the file geometry.

- Fix Geometries: Go to the Toolbox and search for “Fix Geomtries”

- Select Shapefile to Fix: Select Parks file and then Linework to fix this part of the geometry (the other option is structure which may need fixing but not in this case)

- Click Run

## Step 5: Performing a Spatial Join with the Parks Shapefile

The goal is to attach census tract attributes to each park based on their spatial location.

- Open the Spatial Join Tool:

  - In the Processing Toolbox, search for Join attributes by location.

  - You might see two versions:

    - Join attributes by location (summary)

    - Join attributes by location

  - For a one-to-one join (each park gets the attributes of the tract it is within), select Join attributes by location.

- Set Join Parameters:

  - Target Vector Layer: Set this to the clipped NYC Census Tracts layer layer (the layer that will receive additional attributes).

  - Join Vector Layer: Set this to the Fixed Parks layer (the layer that contains the attributes to be joined).

  - Geometric Predicate: Choose within (this means the tool will join attributes to a park if it is within a census tract).

  - Fields to Copy: (Optional) You can select specific fields from the census tracts if you do not need all attributes.

  - Output Layer: Choose to save the joined result as a temporary layer or specify an output file (e.g., nyc_parks_joined.shp).

- Execute the Join:

  - Click Run.

  - A new layer will appear where each park feature now contains attributes from the census tract in which it falls.

- Verify the Join:

  - Right-click the new joined layer and select Open Attribute Table.

  - Scroll through to ensure that fields from the census tracts have been appended to the park records.

# Step 7: Counts of Parks by Census Tracts

We will calculate the total numbers of parks within each census tract here.

## 7.1 Use “Join attributes by location (summary)”

- Open Processing Toolbox

- Search for: Join attributes by location (summary)

### Set Parameters:

- Target Vector Layer:
→ Clipped NYC Census Tracts

- Join Vector Layer:
→ NYC Parks (fixed geometry version)

- Geometric Predicate:
→ intersects

- Fields to Summarize:
→ leave open

- Summaries to Calculate:
→ Select counts

# Step 8: Summing Park Acres by Census Tract

We will calculate the total park acreage within each census tract as another method.

## 8.1 Use “Join attributes by location (summary)”

- Open Processing Toolbox

- Search for: Join attributes by location (summary)

### Set Parameters:

- Target Vector Layer:
→ Clipped NYC Census Tracts

- Join Vector Layer:
→ NYC Parks (fixed geometry version)

- Geometric Predicate:
→ intersects

- Fields to Summarize:
→ Select acres

- Summaries to Calculate:
→ Select sum

- Output Layer:
Choose:

- Save to File…

Name it:

nyc_tracts_acres_sum.shp

Click Run.

# Step 8.2: Export the Layer (If You Saved as Temporary)

If you accidentally saved as a temporary layer:

- Right-click the new layer in the Layers Panel.

- Select:

- Export → Save Features As…

- Choose format:

- ESRI Shapefile

- Name it:

- nyc_tracts_acres_sum.shp

- Click OK.

# Step 9: Replace NULL Acres with Zero

Some tracts will show NULL if they contain no parks. We will convert those to 0.

## 9.1 Create a New Field

- Open the Attribute Table

- Click Field Calculator

- Create a new field:

  - Field Name:

  - acres_sum2

  - Type: Decimal Number (Real)

  - Precision: 2

Use this expression:

CASE WHEN "sum_acres" IS NULL THEN 0 ELSE "sum_acres" END

Click OK.

# Step 9.2: Save Edits Permanently

- Click the Toggle Editing (pencil icon) to stop editing.

- Click Save Edits.

- Confirm changes.

If needed, right-click the layer and choose:

- Save

# Step 10: Select Tracts with More Than One Park

### 10.1 Select the Features

- Open the Attribute Table of your census tract layer with nyc_parks_counts layer.

- Click Select by Expression.

- Use the following expression:

coalesce("count", 0) > 1 or “count” > 1

- Click Select Features.

- Close the Select by Expression window.

The selected tracts now represent census tracts with more than one park.

# Step 10.2: Export Selected Features (IMPORTANT)

To permanently save this subset:

- Right-click the census tract layer in the Layers Panel.

- Select:

- Export → Save Selected Features As…

- In the Format dropdown, choose:

- ESRI Shapefile

- Click Browse and name the file:

- nyc_tracts_more_than_one_park.shp

- Click OK.

# Step 11: Add NYC Borough Boundaries (Clean Frame)

- Right-click NYC Borough Boundary

- Click Properties → Symbology

- Click the symbol

Set:

- Fill Style → No Brush

- Stroke Color → Black

- Stroke Width → 0.8–1.0 mm

Click OK

Place borough layer above tracts in the Layers Panel.

# Step 12: Map Tracts with >1 Park vs. 0 or 1 Park

## 12.1 Style the Full NYC Census Tract Clipped Layer (0 or 1 Park)

- Right-click the full census tract layer.

- Select Properties → Symbology.

- Keep Single Symbol selected.

- Click the symbol.

Set:

- Fill Color → Light Grey
Example: RGB 210, 210, 210

- Stroke Color → Light Grey or thin black

- Stroke Width → 0.2–0.4 mm

Click OK.

This layer will represent all tracts with none or one park.

## 12.2 Style the “More Than One Park” Layer

- Right-click:

- nyc_tracts_more_than_one_park.shp

- Select Properties → Symbology.

- Keep Single Symbol.

- Click the symbol.

Set:

- Fill Color → Green
Example: RGB 34, 139, 34

- Stroke Color → S

- lightly darker green or black

- Stroke Width → 0.4–0.6 mm

Click OK.

# 12.3 Alternative (Cleaner Two-Category Map)

If you want only two categories:

Create a new field called:

multi_park

Field Calculator expression:

CASE WHEN coalesce("count",0) > 1 THEN 'More than 1 park' WHEN coalesce("count",0) = 0 THEN 'No parks' ELSE 'One park' END

Then:

- Open Symbology

- Choose Categorized

- Value = multi_park

- Click Classify

- Set:

  - No parks → Grey

  - More than 1 park → Green

  - One park → Light neutral

# Step 13: Create a Choropleth Map of Park Acreage (Natural Breaks)

We will map total park acreage per census tract using graduated symbology.

## 1)Open the Symbology Panel

- Right-click your census tract layer (the one containing acres_sum2).

- Select:

- Properties

- Click the Symbology tab.

- Change the renderer from:

- Single Symbol

to:

Graduated

## 2)Set the Choropleth Variable

- Value:
Select:

- acres_sum2

(Use acres_sum2, not sum_acres, so NULLs are already handled.)

## 3) Choose Classification Method

- Mode:
Select:

- Natural Breaks (Jenks)

- Classes:
Set to:

- 5

Natural Breaks groups tracts based on clusters in the data, minimizing within-class variance and maximizing between-class differences.

## 4) Select a Green Color Ramp

Click the Color Ramp dropdown and choose a green ramp.

Recommended:

- Light green → Dark green gradient

If needed:

- Click the ramp

- Choose Create New Color Ramp

- Select a gradient from:

  - Light green (low acres)

  - Dark green (high acres)

## 5) Classify the Data

Click:

Classify

You will now see five acreage ranges.

# 14. Lab Deliverables

You will submit two completed maps as PDF or high-resolution PNG files.

Both maps must include:

- Title

- Legend

- Borough boundaries

- Clear symbology

- Your name

- Date

- Data source (Census + NYC Parks)

# Deliverable 1: Choropleth Map – Total Park Acreage

## Map Requirements

Create a choropleth map showing:

Total park acreage per census tract (acres_sum2)

### Symbology Requirements

- Renderer: Graduated

- Classification: Natural Breaks (Jenks)

- Number of Classes: 5

- Color Ramp: Green gradient

  - Light green = low acreage

  - Dark green = high acreage

# Deliverable 2: Choropleth Map – Park-Rich vs. Park-Limited Tracts

Create a two-category map showing:

- Tracts with more than 1 park

- Tracts with 0 or 1 park

You may use:

- Two layers (recommended)
OR

- A categorized field (multi_park variable)